# First Statewide Surficial Geologic Map of Nebraska Underscores Quaternary Landscape Evolution Across the Great Plains to the Central Lowland

Nolan Barrette1, Samuel A. Johnstone1, R.M. Joeckel2, Paul R. Hanson2

1U.S. Geological Survey, Geosciences and Environmental Change Science Center, MS 980, 1 Denver Federal Center, Denver, CO, USA

2Conservation and Survey Division, School of Natural Resources, University of Nebraska-Lincoln, Lincoln, NE, USA

Article: 2000-4000 words; max 8 figures.

## Abstract

Surficial geologic mapping in Nebraska has been carried out primarily at the 1:24,000 scale since the mid-1990s, although there have also been limited efforts to map generalized Quaternary geology within the state and the region. We compiled and evaluated disparate maps (1:24,000 to 1:1,000,000) and datasets—including geologic, soil and soil parent material, and geomorphic maps as well as LiDAR data—to produce a single 1:500,000 scale surficial geologic map of Nebraska. This map satisfies the need for a coarse scale surficial geologic map to be incorporated in a nation-wide U.S. Geological Survey Quaternary geologic map (Barrette et al., 2025; Johnstone et al., 2025). Furthermore, this new map underscores the variability and uniqueness of physical landscapes in Nebraska and portends a richer interdisciplinary understanding of regional geomorphology in agriculture land use and the maintenance of plains ecosystems in the heart of North America.

## Introduction

Nebraska was the nexus of continental changes in fluvial, eolian, and glacial systems during the Quaternary, although the regional physiography and drainage patterns had been developing since the retreat of the Western Interior Seaway in the earliest Paleocene and eolian deposition may have begun as early as the late Eocene (WIS and WRG citations from Matt). The ample and world-renowned

Figure 1. Sketch modified from Swinehart et al. (1994) showing postulated drainage patterns from late Pliocene to late Pleistocene. Solid lines show the main ancestral drainage of the Platte River. Heavy doted lines represent terminal moraine positions of undifferentiated pre-Illinoian glacial maxima.

Quaternary sediments in the state are constrained by a geochronologic framework (e.g. Boellsdorff, 1978; Roy et al., 2004; Balco et al., 2005; Miao et al., 2007; Mason et al., 2011 that have been applied to the evolution of its iconic landscapes. The Platte River system left behind a considerable alluvial sedimentary record and served as a sediment source for the state’s extensive eolian deposits. The Todd Valley, the Rainwater Basin Plains, and other landscapes in the state owe their origins in large measure to the changing Platte River. Similarly, Nebraska has several dunefields, chief among them being the Nebraska Sand Hills, the largest sand sea in the western Hemisphere, but dunes are also found along the state’s major rivers (e.g. Hanson et al., 2009; Puta et al., 2013). Downwind of the Nebraska Sand Hills lie unique landscapes developed on the thickest (as much as 50 m) late Quaternary loess deposits in the United States (Mason et al., 2011; Muhs et al., 2008). Finally, the western limit of multiple pre-Illinoian advances of the Laurentide Ice Sheet extends across eastern Nebraska that left behind local landscapes that can be related directly to that major physiographic boundary.

## Previous Surficial Geologic Mapping in Nebraska

The most recent state-wide geologic map of Nebraska only depicts bedrock (Burchett et al., 1986), but Joeckel et al., (2018) estimated that more than 87% of the state is covered by post-Miocene surficial sediments, almost exclusively Quaternary in age. Existing maps of Quaternary deposits do not have statewide coverage, and they have been mapped using different conventions (e.g. composite mapping units in some cases), and at scales ranging from 1:62,500 to 1:5,000,000 ( Scott, 1978; Diffendal, 1991; Hallberg et al., 1991; Swinehart et al., 1994; Swinehart et al., 1994; Swinehart and Diffendal, 1995, 1997; Soller and Reheis, 2004; Diffendal et al., 2008; Horton et al., 2017; Fig. 1). Some of these maps also illustrate stark contrasts in available data across 102° west longitude (Soller and Reheis, 2004) or show discrepancies in mapping units across state boundaries  (Horton et al., 2017). Since the mid-1990s the CSD’s STATEMAP program, funded by the U.S. Geological Survey, emphasized true surficial

Figure 2. Locations of previous mapping used in compiling the Quaternary Geology of Nebraska.

geologic mapping at 1:24,000 scale, including in the glaciated portion of the state (e.g. Joeckel and Howard, 2009;  Young et al., 2009), and the Niobrara (Joeckel et al., 2013; 2016) and Platte (e.g. Mason and Joeckel, 2001; Condon, 2005; Hanson et al., 2024) River valleys and the Panhandle (LaGarry et al., 2005a; 2005b), but the majority of the state has not been mapped at this scale. Our new 1:500,000 surficial geologic map, which employs the U.S. Geological Survey’s Geologic Mapping Schema (GeMS) standard, addresses some of these issues and sets the stage for future work.

## Map Data Schema

We applied the Geologic Mapping Schema (GeMS) standard digital data format for this map (USGS, 2020). Hereafter, we refer to data in GeMS format as tables in bold text and fields in italics. Our six spatial layers are ContactsAndFaults, MapUnitPolys, GeologicLines, CartographicLines, DataSourcePolys and OverlayPolys. ContactsAndFaults holds all the vector line features that bound map-unit polygons; including contacts, faults (non-existent in this dataset), boundaries of water bodies, and map boundaries. MapUnitPolys contains all of the map-unit polygons, including water. These polygons are all bounded by the features within ContactsAndFaults. In places throughout the map there are important relationships with underlying, and in some cases intermittently exposed, geology that we depict with stacked unit labels. In these regions a polygon retains its primary map unit, but has a label that indicates what underlying unit(s) are present (e.g., Ql/Qat1 indicates unit Qat1 is present beneath unit Ql, and may occasionally be exposed at the surface within a polygon). This stack unit strategy particularly helps highlight geologic relationships that are otherwise buried by extensive eolian deposits.  Regions of intermittent exposure of underlying bedrock units are stored in OverlayPolys, which also captures regions of approximate Peoria Loess thickness as polygons.  GeologicLines contains any lines that are associated with geologic features, but do not define a boundary between map-unit polygons. CartographicLines contains waterways in the state. DataSourcePolys contains the spatial footprints of each of the references utilized in this mapping effort. Our dataset also contains four non-spatial tables: DataSources, DescriptionOfMapUnits, GeoMaterialDict, and Glossary. DataSources identifies the references for the content of the geologic map database and ties each individual spatial feature to one or more data sources by way of a pipe-delimited (“|”) DataSourcesID foreign key to the Datasources_ID field in DataSources. Where multiple delimited data sources are referenced by a feature they are listed in order of importance on the mapped feature. DescriptionOfMapUnits contains map unit descriptions. It is related to the MapUnitPolys by way of a MapUnit primary and foreign key. GeoMaterialDict provides definitions and a hierarchy for GeoMaterial names within the GeMS schema, which is an important part of the detailed descriptions held within the DMU. Glossary stores definitions for the terminology used throughout the GeMS geodatabase. We employed the Federal Geographic Data Committee (FGDC) Digital Cartographic Standard for Geologic Map Symbolization (FGDC, 2006). These symbols are stored in the Symbol field in each feature class or non-spatial table and are a key to a specific cartographic symbol in the FGDC standard.

## Spatial Data Compilation

To produce this map, we relied on various pre-existing maps and other data sources (Fig. 2; Table 1). We began compiling our map with a generalized 1:1,000,000 scale map of Nebraska soils (CSD, 1990) which was the only existing statewide product approximating surficial geology. This map describes 80 unique soil associations, which we grouped into nine categories based on their parent materials, namely. We dissolved the original polygons to create a first iteration of MapUnitPolys and ContactsAndFaults, to which we appended a map of  dune types found in the Nebraska Sand Hills (Swinehart, 1994).	Subsequently we refined the location of major contacts, discussed more below, by primarily focusing in areas lacking detailed maps (Fig. 2). These major contacts are mostly margins of alluvial deposits and terraces inset into valleys that have been previously defined by the statewide soils map (CSD, 1990).  Hillshades and other elevation derivatives from USGS 3DEP data (U.S. Geological Survey, 2022) were used to relocate contacts relative to topography (Fig. 4).

Figure 4. A region north of the confluence of the Elkhorn and Platte Rivers showing original contacts from (NCSD Soils) in red and our newly mapped contacts in black, both overlaying a hillshade from U.S. Geological Survey, (2022).

Where previous mapping of an appropriate scale and content was not available, we manually generalized Quaternary geology based on previous maps and classifications of soil surveys based on parent material. In completing this original mapping, we had a target scale of 1:500,000 and sought to capture to capture relevant map patterns and topographic relationships.  This involved smoothing line work and aggregating previously mapped polygons to reflect generalized stratigraphy and spatial geometry (Fig. 5), as well as extending the boundaries of previously mapped units beyond the limits of the source materials. We performed several iterations of this process driven through the examination of map-unit margins relative to elevation derivatives and remotely sensed data.

Figure 5. A region near Scottsbluff, NE, illustrating our mapping decisions compared to those of the source map. Panel A shows the 1:250,000 scale original map data from Swinehart and Diffendal Jr., (1997), overlaying the topographic data from U.S. Geological Survey,  (2022) shown in panel B. Panel C shows our new mapping similarly overlaying the topographic data in panel B.

Figure 6 illustrates the spatial distribution of reproduced and modified linework described above. The map representation of the Sand Hills is effectively unchanged. Parts of the northwestern, far western, and west-central Nebraska were previously mapped in greater detail than 1:500,000 (Fig. 2). We performed extensive manual generalization in these areas, which include major fluvial systems and their deposits. We also continued our linework along such features from existing maps using topographic relationships and soil maps, resulting in modified and/or original linework that is consistent with topography along all major river valleys.

Figure 6. Linework modification. Grey lines show new linework based only on U.S. Geological Survey, (2022) topographic data. Orange lines show linework that has been modified from the original data sources. Green lines show linework that has been directly taken from original data sources.

As referenced above, we directly incorporated digital data from Swinehart et al., (1994) and Hallberg et al., (1994) to depict the extent of pre-Illinoian glaciers and Mason (2001) to depict Peoria loess thickness.

## Synthesis of Geology

Although we began with a generalized soils map and considered broad soil parent material associations (alluvium, eolian sediments, glacial sediments, and bedrock), we also employed existing geologic maps, LiDAR data, known sediment landform assemblages determined by previous 1:24,000 mapping, a map of dune types in the Sand Hills (Swinehart, 1994) and information from web soil survey (Soil Survey Staff, 2023) in successive iterations of map development (Table 1). Unsurprisingly, most of the alluvial units that we map are along major rivers and certain creeks with broad alluvial valleys. Where defensible at this scale of resolution, and given existing data, we mapped subdivisions of alluvium in terms of relative age (e.g., recent, intermediate, and older). Eolian sediments are represented as either (1) units unto themselves (e.g. dune sands), (2) parts of composite mapping units (e.g. loess over alluvium), or (3) in combination with other closely associated eolian sediments (e.g. loess and eolian sands). Pre-Illinoian glacial sediments, chiefly tills, are the dominant components in our map unit “Qt” which categorizes a part of southeastern Nebraska in the Dissected Till Plains. The Wisconsinan loess (Peoria Loess and Gilman Canyon Formation) mantles ridgetops, flat summits, and some slopes in this region. In the remainder of the glaciated portion of eastern Nebraska we employ mapping unit Ql, which represents thicker Wisconsinan, late Illinoian, and even older loess, atop pre-Illinoian till. Where there is no significant Quaternary mantle, we categorize older “bedrock” units into broad chronostratigraphic categories (e.g., Tertiary (T) and Cretaceous (K)).

## Interpolation of loess thickness

Table 1. Crosswalk table showing relationships between original source classifications and our synthesis map units. The ‘MapUnit’ column refers to the map unit code assigned to each unit. The ‘Name’ column refers to the full unit name. The ‘Description’ column provides more detail on what the named unit includes. The ‘Original Classification’ refers to the original description of the unit from the original source, denoted in ‘Original Source’,

## Discussion and Conclusions

Our completed map (Fig. 7) is distinct from previous maps in its scale (1:750,000), statewide representation, adherence to the essence of surficial geologic mapping, and matching of mapping unit composition and resolution to the selected scale. Given the scale of our map, we only mapped bedrock in a few areas, and at that chiefly the upper Paleogene and Neogene strata that are exposed, or very close to the land surface, in large parts of the Panhandle. There are, indeed, comparatively small exposures of bedrock along major drainages in northern and eastern Nebraska, but it is inappropriate to map them at the 1:750,000 scale that we used. The large area occupied by the Sand Hills and the extensive distribution of loess deposits dictate that Quaternary eolian sediments cover >70% of the mapped area. Clearly, the greatest extent of Quaternary fluvial sediments is in the valleys of the North Platte, South Platte and Platte rivers. Although pre-Illinoian glacial sediments underlie the eastern quarter of Nebraska are prominent in only one of our mapping units (Qt) in the southeastern part of the state.

Our map is a consistent depiction of Nebraska’s surficial geology, and particularly its Quaternary deposits, but it is only a starting point for future refinements in mapping as well as Quaternary stratigraphy and geochronology. Most of the data sources for our map are decades old. New data and interpretations would doubtless improve our understanding of the geologic evolution of Nebraska in the past ~2.6 million years. Existing 1:250,000 scale maps of the state prioritize pre-Quaternary bedrock units, so surficial geologic maps should be produced at this scale. Revisit suggestions in Joeckel et al., 2019.

## Acknowledgements

Margaret Berry and C.A. Ruleman provided reviews on an early draft of this map that helped tremendously in guiding the work. Primary support for this work came from the FEDMAP component of the US Geological Survey National Cooperative Geologic Mapping Program. CSD acknowledges the financial support from the US Geological Survey’s STATEMAP program that funded surficial geologic mapping efforts in the state since the mid-1990s. This draft manuscript is distributed solely for purposes of scientific peer review. Its content is deliberative and pre-decisional, so it must not be disclosed or released by reviewers. Because the manuscript has not yet been approved for publication by the U.S. Geological Survey (USGS), it does not represent any official USGS finding or policy. Any use of trade, firm, or product names is for descriptive purposes only and does not imply endorsement by the U.S. Government.

Figure 7. Quaternary geologic map of Nebraska. Insets show the loess thickness in meters and the index to mapping sources.

# References

Aber, J.S., 1999, Pre-Illinoian glacial geomorphology and dynamics in the central United States, west of the Mississippi, in Glacial Processes Past and Present, Geological Society of America, doi:10.1130/0-8137-2337-X.113.

Balco, G., Stone, J.O.H., and Mason, J.A. 2005. Numerical ages for Plio-Pleistocene sediment sequences by 26Al/10Be dating of quartz in buried paleosols. Earth and Planetary Science Letters 232, 179-191.

Barrette, N.C., Johnstone, S.A., Campos, J.-M., Colgan J.P., and Roe, W.P., 2025, Cooperative National

Geologic Map—Quaternary geology: U.S. Geological Survey data release, https://doi.org/10.5066/P16SJAC6.

Boellstorff, J., 1978, North American Pleistocene Stages reconsidered in light of probable Pliocene-

Pleistocene continental glaciation. Science, vol. 202, p. 305-307.

Condon, S.M., 2005, Geologic Studies of the Platte River, South-Central Nebraska and Adjacent Areas—Geologic Maps, Subsurface Study, and Geologic History: U.S. Geological Survey Professional Paper 1706, Plates 1 & 2, https://pubs.usgs.gov/pp/pp1706/.

Diffendal Jr., R.F., 1991, Geologic map showing configuration of the bedrock surface, North Platte, 1o x 2o quadrangle, Nebraska: U.S. Geological Survey Miscellaneous Investigations Series Map I-2277.

Diffendal Jr., R.F., Voorhies, M.R., Voorhies, E.J., LaGarry, H.E., Timperley, C.L., and Perkins, M.E., 2008. Geologic Map of the O'Neill 1o X 2o Quadrangle, Nebraska, with Configuration Maps of Surfaces of Formations. Conservation and Survey Division, Institute of Agriculture and Natural Resources, University of Nebraska-Lincoln Geologic Map GMC-34, https://digitalcommons.unl.edu/natrespapers/304.

Feng, Z.-D., Johnson, W.C., and Diffendal Jr., R.F., 1994, Environments of Aeolian Deposition in South-Central Nebraska During the Last Glacial Maximum. Physical Geography, v. 15, p. 249–261, doi:10.1080/02723646.1994.10642515.

FGDC, 2006, FGDC Digital Cartographic Standard for Geologic Map Symbolization: Federal Geographic Data Committee Document Number FGDC-STD-013-2006, 290 p., https://ngmdb.usgs.gov/fgdc_gds/.

Finnegan, N.J., 2013, Interpretation and downstream correlation of bedrock river terrace treads created from propagating knickpoints: Journal of Geophysical Research: Earth Surface, v. 118, p. 54–64, doi:10.1029/2012JF002534.

Flanagan, K.M., and Montagne, J., 1993, Neogene stratigraphy and tectonics of Wyoming, in Snoke, A.W., Steidtmann, J.R., and Roberts, S.M., editors, Geology of Wyoming: Geological Survey of Wyoming Memoir No. 5, p. 572–607.

Hallberg, G.R. et al., 1991, Quaternary geologic map of the Des Moines 4° x 6° Quadrangle, United States: U.S. Geological Survey Miscellaneous Investigations Series Map I-1420 (NK-15), https://ngmdb.usgs.gov/Prodesc/proddesc_9217.htm.

Hanson, P.R., Joeckel, R.M., Young, A.R., Horn, J., 2009, Late Holocene Dune Activity in

the Eastern Platte River Valley, Nebraska, Geomorphology 103, 555-561.

Hanson, P.R., Shrestha, N., Girard, M., Joeckel, R.M., Reinier, C.L. 2024. Surficial geology

of the Central City West 7.5’ Quadrangle, Nebraska. Conservation and Survey Division, University of Nebraska-Lincoln.

Heckel, P.H., 2008, Pennsylvanian cyclothems in Midcontinent North America as far-field effects of waxing and waning of Gondwana ice sheets, in Special Paper 441: Resolving the Late Paleozoic Ice Age in Time and Space, Geological Society of America, v. 441, p. 275–289, doi:10.1130/2008.2441(19).

Horton, J.D., San Juan, C.A., and Stoeser, D.B., 2017, The State Geologic Map Compilation (SGMC) geodatabase of the conterminous United States: USGS? Data Series Report 1052, 56 p., doi:10.3133/ds1052.

Joeckel, R.M., Diffendal, R.F., Hanson, P.R., Korus, J.T., 2018. Geologic Mapping of

Nebraska: Old Rocks, New Maps, Fresh Insights. Great Plains Research 28, 119-147.

Joeckel, R.M., Howard, L.M., 2009. Surficial Geology of the Hallam 7.5 Minute Quadrangle,

Nebraska. Conservation and Survey Division, University of Nebraska-Lincoln.

Joeckel, R.M., Tucker, S.T., Howard, L.M., Kuzila, M.S., 2013. Surficial Geology of the Naper

7.5 Minute Quadrangle, Nebraska. Conservation and Survey Division, University of Nebraska-Lincoln.

Joeckel, R.M., Howard, L.M., Tucker, S.T., Nuzzo, G., 2016. Surficial Geology of the Norden

7.5 Minute Quadrangle, Nebraska. Conservation and Survey Division, University of Nebraska-Lincoln.

Johnstone, S.A., Colgan, J.P., and Roe, W.P., 2025, Geospatial database for the cooperative national

geologic maps: U.S. Geological Survey data release, https://doi.org/10.5066/P1DC4XFG.

LaGarry, H.E., Swinehart, J.B., and LaGarry, L., 2005a, Surficial Geology of the Hay Springs

NE 7.5 Minute Quadrangle, Nebraska: University of Nebraska Conservation and Survey Division, scale 1:24,000.

LaGarry, H.E., Swinehart, J.B., and LaGarry, L., 2005b, Surficial Geology of the Whiteclay SE

7.5 Minute Quadrangle, Nebraska: University of Nebraska Conservation and Survey Division, scale 1:24,000.

Korus, J.T., Joeckel, R.M., Abraham, J.D., Høyer, A.-S., and Jørgensen, F., 2021, Reconstruction of pre-Illinoian ice margins and glaciotectonic structures from airborne ElectroMagnetic (AEM) surveys at the western limit of Laurentide glaciation, Midcontinent U.S.A.: Quaternary Science Advances, v. 4, p. 100026, doi:10.1016/j.qsa.2021.100026.

Maat, P.B., and Johnson, W.C., 1996, Thermoluminescence and new 14C age estimates for late quaternary loesses in southwestern Nebraska: Geomorphology, v. 17, p. 115–128, doi:10.1016/0169-555X(95)00099-Q.

Mason, J.A., Swinehart, J.B., Hanson, P.R., Loope, D.B., Goble, R.J., Miao, X., Schmeisser, R.,

2011, Late Pleistocene dune activity in the central Great Plains, USA, Quaternary Science Reviews 30, 3858-3870.

Mason, J.A., R.M. Joeckel, 2001. Surficial Geologic Map of the Ashland East 1:24,000

Quadrangle. Nebraska Conservation and Survey Division.

Mears, B., Jr., 1993, Geomorphic history of Wyoming and high-level erosion surfaces, in Snoke, A.W., Steidtmann, J.R., and Roberts, S.M., editors, Geology of Wyoming: Geological Survey of Wyoming Memoir No. 5, p. 609–626.

Miao, X., Mason, J.A., Swinehart, J.B., Loope, D.B., Hanson, P.R., Goble, R.J., and Liu, X., 2007, A 10,000 year record of dune activity, dust storms, and severe drought in the central Great Plains: Geology, v. 35, p. 119, doi:10.1130/G23133A.1.

Muhs, D.R., Aleinikoff, J.N., Stafford, T.W., Jr, Kihl, R., Been, J., Mahan, S.A., and Cowherd, S., 1999, Late Quaternary loess in northeastern Colorado: Part I—Age and paleoclimatic significance: GSA Bulletin, v. 111, p. 1861–1875, doi:10.1130/0016-7606(1999)111<1861:LQLINC>2.3.CO;2.

Muhs, D.R., and Budahn, J.R., 2019, New geochemical evidence for the origin of North America’s largest dune field, the Nebraska Sand Hills, central Great Plains, USA: Geomorphology, v. 332, p. 188–212, doi:10.1016/j.geomorph.2019.02.023.

Muhs, D.R., Reynolds, R.L., Been, J., and Skipp, G., 2003, Eolian sand transport pathways in the southwestern United States: importance of the Colorado River and local sources: Quaternary International, v. 104, p. 3–18, doi:10.1016/S1040-6182(02)00131-3.

Muhs, D.R., Stafford, T.W., Swinehart, J.B., Cowherd, S.D., Mahan, S.A., Bush, C.A., Madole, R.F., and Maat, P.B., 1997, Late Holocene Eolian Activity in the Mineralogically Mature Nebraska Sand Hills: Quaternary Research, v. 48, p. 162–176, doi:10.1006/qres.1997.1913.

Muhs, D.R., Swinehart, J.B., Loope, D.B., Been, J., Mahan, S.A., and Bush, C.A., 2000, Geochemical Evidence for an Eolian Sand Dam across the North and South Platte Rivers in Nebraska: Quaternary Research, v. 53, p. 214–222, doi:10.1006/qres.1999.2104.

Puta, R.A., Hanson, P.R., Young, A.R., 2013, Late Holocene history of dune activity along

the Elkhorn River in northeastern Nebraska, Great Plains Research 23, 11-24.

CSD 24k, Mutliple Publication Dates, 7.5" Geologic maps published by the Nebraska Conservation and Survey Division in Northwest Nebraska:, https://snr.unl.edu/csd/geology/statemap.aspx.

Roy, M., Clark, P.U., Barendregt, R.W., Glasmann, J.R., Enkin, R.J., 2004. Glacial stratigraphy and

paleomagnetism of late Cenozoic deposits of the north-central United States. Geological Society of America Bulletin 116, 30-41.

Swinehart J.B., 1994. Wind-blown Deposits. In: Bleed AS and Flowerday CA (eds) An Atlas of

the Sand Hills. Resource Atlas 5b. Lincoln: Conservation and Survey Division, University of Nebraska-Lincoln, pp. 17–28.

Conservation and Survey Division, 1990. General Soil Map of Nebraska, 1:1,000,000. https://snr.unl.edu/data/geographygis/soil.aspx.

Reed, E.C., and Dreeszen, V.H., 1965, Revision of the Classification of the Pleistocene Deposits of Nebraska:

Roberts, L.N.R., and Kirschbaum, M.A., 1995, Paleogeography of the Late Cretaceous of the Western Interior of Middle North America - Coal Distribution and Sediment Accumulation: U.S. Geological Survey Professional Paper 1561,.

Scott, G.R., 1978, Map showing geology, structure, and oil and gas fields in the Sterling 1 degree x 2 degrees quadrangle, Colorado, Nebraska, and Kansas, U.S. Geological Survey, Miscellaneous Investigations Series Map I-1092, 1:250,000.

Soil Survey Staff, Natural Resources Conservation Service, United States Department of Agriculture. Web Soil Survey. Available online at https://websoilsurvey.nrcs.usda.gov/. Accessed [02/01/2023].

Soller, D.R., and Reheis, M.C., 2004, Surficial materials in the conterminous United States: U.S. Geological Survey Open-file Report 03-275.:

Souders, V.L., 2000, Geologic maps and cross sections showing configurations of bedrock surfaces, Broken Bow 1° x 2° quadrangle, east-central Nebraska: U.S. Geological Survey Geologic Investigations Series I–2725.

Swinehart, J.B., and Diffendal Jr., R.F., 1997, Geologic map of the Scottsbluff 1° x 2° quadrangle, Nebraska and Colorado: U.S. Geological Survey IMAP 2545, https://doi.org/10.3133/i2545.

Swinehart, J.B., Dreeszen, V.H., Richmond, G.M., Tipton, M.J., Bretz, R., Steece, F.V., Hallberg, G.R., and Goebel, J.E., 1994, Quaternary geologic map of the Platte River 4° x 6° quadrangle, United States: U.S. Geological Survey U.S. Geological Survey Miscellaneous Investigations Series Map I-1420 (NK-14), https://ngmdb.usgs.gov/Prodesc/proddesc_9216.htm.

U.S. Geological Survey, 2022, 1 Arc-second Digital Elevation Models (DEMs) - USGS National Map 3DEP Downloadable Data Collection:, https://www.usgs.gov/3d-elevation-program.

USGS, 2020, GeMS (Geologic Map Schema)—A standard format for the digital publication of geologic maps: Techniques and Methods Report 11-B10, doi:10.3133/tm11B10.

Wayne, W.J., 1985, Drainage Patterns and Glaciations in Eastern Nebraska. TER-QUA Symposium Series 1, 111-117.

Young, A.R., Korus, J.T., Howard, L.M., Waszgis, M.M., 2016. Surficial Geology of the Raymond 7.5 Minute

Quadrangle. Conservation and Survey Division, University of Nebraska-Lincoln.